Multi-core:

Array_dim: 10	-> t_duration= CDF
Array_dim: 100	-> t_duration= 5C02
Array_dim: 1000	-> t_duration= 2FBC7

Single-core:

Array_dim: 10	-> t_duration= E3D
Array_dim: 100	-> t_duration= 81B3
Array_dim: 1000	-> t_duration= 49DB0

Multi-core SIMD + optimization:

Array_dim: 10	-> t_duration= 33F
Array_dim: 100	-> t_duration= E63
Array_dim: 1000	-> t_duration= 467F

Single-core SIMD + optimization:

Array_dim: 10	-> t_duration= 287
Array_dim: 100	-> t_duration= 164B
Array_dim: 1000	-> t_duration= 75AD

In general, the parallel execution version provides a better performance (in terms of execution time) with respect to the single-core version. However, for smaller array dimensions (like 10 in our case), the overhead implied for managing the parallel processing results being more costy than the benefits the parallel architecture gives, making the single-core architecture faster than the parallel one for this specific case.

In general, enabling optimization options (O2) and applying vectorization (SIMD) allow the performance (in terms of execution time) to increase on both single-core and parallel architecture with the parallel version always performing better than the single-core for larger arrays.

Moreover, there is a difference in speedup contribute between that provided by the change of architecture and the vectorization enabling, which sees the vectorization version giving a higher contribute than the parallel one.